ALS 100 Plus+

Integrated Audio Controller

w/ 3D Sound

Revision 1.10

January 20, 1997

Copyright © 1996 by Avance Logic, Inc.  All rights reserved

This document is the property of Avance Logic, Inc.  No copies shall be made of its contents in whole or in part, nor shall the contents be disclosed to any party not previously and expressly authorized by Avance Logic, Inc.

The information contained in this document is subject to change without notice.  Avance Logic, Inc. shall not be held liable for any damages attributable to subsequent changes to the contents of this document.

The technical specifications contained in this document are based on computer simulations and not on actual measurements.  These specifications are subject to change and will be updated as required by revisions to this document.

Windows and Windows Sound System are registered trademarks of the Microsoft Corporation.  OS/2 Warp is a registered trademark of the International Business Machines Corporation.  Sound Blaster, Sound Blaster Pro, and Sound Blaster 16 are registered trademarks of Creative Labs.  Adlib is a registered trademark of the Adlib Corporation.  All other brand and product names are trademarks of their respective owners.

This product is intended for use in normal commercial applications.  Use in an environment other than that listed herein is not recommended without prior approval by Avance Logic, Inc.

Table of Contents

µ1.0 Introduction	§

2.0 Features	§

3.0 Module I/O Pins	§

4.0 Electrical Specification	§

Mixers	§

Maximum Ratings	§

Timing Parameters	§

Operating Environment	19

5.0  Physical Description	§

6.0  Design Example	24

1.0	Introduction

Sound system designs based on the ALS100Plus+ Integrated Audio Subsystems are true Plug and Play systems.  ALS100Plus+ Integrated Audio Subsystems bring forth a new age of highly integrated, single chip sound products.  They are cost-efficient audio solutions for motherboard and add-in card implementations.  Avance Logic, Inc. has taken the approach of integrating the following functions into a single ASIC.

Subsystem control logic

Digital to analog and analog to digital converters

Mixer functions

3-D Sound Effect Processor

CD-ROM controller(ATAPI IDE)

Enhanced game/MIDI port

By bringing these functions into a single ASIC, the overall product cost is reduced.  Additionally, reliability and compatibility are improved by the reduced number of components required to build a sound system.

ALS100Plus+ Integrated Audio Subsystems fully support the ISA Plug and Play standard.  For non-ISA Plug and Play compatible systems, a software utility is available that sets the IRQ and DMA resources.  Add-in sound cards and motherboard implementations designed around the ALS100Plus+ Integrated Audio Subsystems are completely jumper and switch free.

ALS100Plus+ Integrated Audio Subsystems support the following functions:

Audio Input

CD-ROM Audio

Monaural In

Stereo Line In

Microphone In

Audio Output

Stereo Line Out

Mono/modem Out

Enhanced game/MIDI Port

External MIDI input and output

Joystick input

CD-ROM Controller

ATAPI IDE

I2S/YAC512 Interface

FIGURE 1.1		ALS100Plus+ FUNCTIONAL BLOCK DIAGRAM

MIC

ISA CONTROL

LOGIC

CODEC/

MIXER

LINE-L/R

-SIOR 														CDL/R

-SIOW														LINE-L/R

SAEN														MUSIC-L/R

SA[15:0] 														MONO-O

SD[7:0] 														MONO-I

XTALI 													 	ALEFT/

3D Effect Processor

ENGINE

RESET  														ARIGHT

SERIAL/AUDIO

SLRCK

SIGNAL

PROCESSOR

SDATA

SCLK

INTERRUPT

IRQ 9/5

MIDI PORT

/7/10/11													    MIDIIN

DMA LOGIC

-DACK0/														MIDIOUT

MODEM

INTERFACE

1/3															-COMCS

DRQ0/ 														COMIRQ

GAME PORT

1/3															IRQ3/4

GD[7:0]

CD-ROM LOGIC

-SEL17X                                           				 -CDDENL

-SEL376    													 -CDDENH

-XRST

2.0 Features

Single, mixed-signal, high performance VLSI sound ASIC

Compatible with Sound Blaster™, Sound Blaster Pro™, Sound Blaster 16 Emulation and Windows Sound System™

Built-in 3D Sound Effect Processor

External FM/Wavetable Synthesizer Support

ISA Plug and Play support logic

External PNP modem chip interface

I2S interface support

Supports Sound Blaster ADPCM decompression

MPU-401 MIDI controller with FIFO

Supports 8-bit Type F DMA playback

Software selectable DMA lines(0, 1, 3)

Software selectable interrupt lines(5, 7, 9, 10, 11)

8/16 bit mono/stereo digital audio from 4 kHz to 48 kHz

8/16 bit CD-ROM interface

Enhanced game port support

MIDI port with input and output FIFO

Full duplex for concurrent recording and playback

16 bit PNP address I/O decoder

5.0V digital/analog mode

100 pin PQFP package

Pin to pin compatible with ALS200

3.0  Module I/O Pins

- ISA Address

- Item

- Pin Name

- Type

- Pin No.

- Description

- 1

- SA0

- I

- 92

- ISA SYSTEM ADDRESS BIT 0

- 2

- SA1

- I

- 93

- ISA SYSTEM ADDRESS BIT 1

- 3

- SA2

- I

- 94

- ISA SYSTEM ADDRESS BIT 2

- 4

- SA3

- I

- 95

- ISA SYSTEM ADDRESS BIT 3

- 5

- SA4

- I

- 96

- ISA SYSTEM ADDRESS BIT 4

- 6   SA5

- I

- 97

- ISA SYSTEM ADDRESS BIT 5

- 7

- SA6

I

98

ISA SYSTEM ADDRESS BIT 6

8

SA7

I

99

ISA SYSTEM ADDRESS BIT 7

9

SA8

I

100

ISA SYSTEM ADDRESS BIT 8

10

SA9

I

1

ISA SYSTEM ADDRESS BIT 9

11

SA10

I

2

ISA SYSTEM ADDRESS BIT 10

12

SA11

I

3

ISA SYSTEM ADDRESS BIT 11

13

SA12

I

4

ISA SYSTEM ADDRESS BIT 12

14

SA13

I

5

ISA SYSTEM ADDRESS BIT 13

15

SA14

I

6

ISA SYSTEM ADDRESS BIT 14

16

SA15

I

7

ISA SYSTEM ADDRESS BIT 15

17

SAEN

I

8

ISA DMA ADDRESS ENABLE

ISA Data

Item

Pin Name

Type

Pin No.

Description

18

SD0

I/O

9

ISA SYSTEM DATA BIT 0

19

SD1

I/O

10

ISA SYSTEM DATA BIT 1

20

SD2

I/O

11

ISA SYSTEM DATA BIT 2

21

SD3

I/O

12

ISA SYSTEM DATA BIT 3

22

SD4

I/O

14

ISA SYSTEM DATA BIT 4

23

SD5

I/O

15

ISA SYSTEM DATA BIT 5

24

SD6

I/O

16

ISA SYSTEM DATA BIT 6

25

SD7

I/O

17

ISA SYSTEM DATA BIT 7

ISA Control

Item

Pin Name

Type

Pin No.

Description

26

-SIOW

I

67

ISA I/O WRITE STROBE

27

-SIOR

I

68

ISA I/O READ STROBE

28

RESET

I

19

ISA SYSTEM RESET

29

XTALI

I

74

CRYSTAL/OSCILLATOR IN

30

XTALO

O

75

CRYSTAL OUT

ISA DMA

Item

- Pin Name

- Type

- Pin No. Description

- 31

- DRQ0  hiZ

- 85

- ISA DMA REQUEST 0

- 32

- DRQ1

- hiZ

- 78

- ISA DMA REQUEST 1

- 33

- DRQ3

- hiZ

- 70

- ISA DMA REQUEST 3

- 34

- -DACK0

- I   84

- ISA DMA ACKNOWLEDGE 0

- 35

- -DACK1

- I

- 77

- ISA DMA ACKNOWLEDGE 1

- 36

- -DACK3

- I

- 69

- ISA DMA ACKNOWLEDGE 3

- ISA INTERRUPT

- Item

- Pin Name

- Type

- Pin No.

- Description

- 37

- IRQ2/9

- hiZ

- 66

- ISA INTERRUPT REQUEST 2/9

- 38

- IRQ5

- hiZ

- 80

- ISA INTERRUPT REQUEST 5

39

IRQ7

hiZ

79

ISA INTERRUPT REQUEST 7

40

IRQ10

hiZ

81

ISA INTERRUPT REQUEST 10

41

IRQ11

hiZ

82

ISA INTERRUPT REQUEST 11

42

XCLK

O

83

14.318 MHz System Clock Output

MIDI INTERFACE

Item

Pin Name

Type

Pin No.

Description

43

MIDIIN

I

71

MIDI SERIAL INPUT

44

MIDIOUT

I/O

72

MIDI SERIAL OUTPUT

EXTERNAL SERIAL AUDIO DATA

Item

Pin Name

Type Pin No.

Description

45

SDATA

I

55

YAC512/I2S SERIAL DATA INPUT

46

SLRCK

I

56

YAC512/I2S LEFT/RIGHT CHANNEL

SELECT INPUT

47

SCLK

I

57

YAC512/I2S BIT CLOCK INPUT

MODEM

Item

Pin Name

Type

Pin No.

Description

48

-COMCS/

-BIOW

I/O

59

MODEM CHIP SELECT/BUFFERED

SYSTEM I/O WRITE STROBE

49

COMIRQ/

BA0

I/O

60

MODEM INTERRUPT REQUEST/

BUFFERED SYSTEM ADDRESS 0

50

IRQ3/

BA2

O

62

ISA INTERRUPT REQUEST 3/

BUFFERED SYSTEM ADDRESS 2

51

IRQ4/

BA1

O

61

ISA INTERRUPT REQUEST 4/

BUFFERED SYSTEM ADDRESS 1

ANALOG AUDIO

Item

Pin Name

Type

Pin No.

Description

52

MIC

I

40

MICROPHONE INPUT

53

LINE-L

I

34 EXTERNAL LINE INPUT LEFT

54

LINE-R

I

47

EXTERNAL LINE INPUT RIGHT

55

CDL

I

33

EXTERNAL CD AUDIO INPUT LEFT

56

CDR

I

48EXTERNAL CD AUDIO INPUT RIGHT

57

MUSIC-L

I

36

EXTERNAL WAVETABLE MUSIC

## INPUT LEFT

## 58

## MUSIC-R

## I

- 45

- EXTERNAL WAVETABLE MUSIC

- INPUT RIGHT

- 59

- MONO-I

- I

- 53

- PC SPEAKER/MODEM MONO INPUT

- 60

- MONO-O

- O   41

- PC SPEAKER/MODEM MONO

- OUTPUT

- 61

- ALEFT

- O

- 35

- AUDIO MIXER LEFT OUTPUT

- 62

- ARIGHT

- O

- 46

- AUDIO MIXER RIGHT OUTPUT

- 63

- VREF1

- O

- 42

- 5V REFERENCE VOLTAGE

- OUTPUT

- 64

- VREF2

- O

- 395V REFERENCE VOLTAGE

- OUPUT

- 65

- ADL-O

- I

- 30

- LEFT A/D CONVERTER ANTI-ALIAS

- FILTER

- 66

- ADR-O

- I

- 51

- RIGHT A/D CONVERTER ANTI-

- ALIAS FILTER

- 67

- ADL-I

- I

- 29

- LEFT INPUT MIXER LOOP BACK FILTER

- 68   ADR-I

- I

- 52

- RIGHT INPUT MIXER LOOP BACK FILTER

- 69

- DAL-O

- I

- 31

- LEFT D/A CONVERTER LOW-PASS

- SMOOTH FILTER

- ANALOG AUDIO(CONT.)

- Item

- Pin Name

- Type

- Pin No.

- Description

- 70

- DAR-O

- I

- 50

- RIGHT D/A CONVERTER LOW-

- PASS SMOOTH FILTER

- 71

- DAL-I

- I

- 32

- LEFT D/A CONVERTER LOOP

- BACK FILTER

- 72

- DAR-I

- I   49

- RIGHT D/A CONVERTER LOOP

- BACK FILTER

- 73

- FMDACL-F

- I

- 37

- FM SYNTHESIZER LEFT D/A LOW-

- PASS FILTER

- 74

- FMDACR-F

- I

- 44

- FM SYNTHESIZER RIGHT D/A LOW-

- PASS FILTER

- EXTERNAL GAME PORT, CD-ROM, AND EXPANSION

- Item

- Pin Name

- Type

- Pin No.

- Description

- 75

- GD0

- I/O

- 20

- GAME PORT A TIMER X

- 76

- GD1

- I/O

- 21

- GAME PORT A TIMER Y

- 77

- GD2

- I/O

- 22

- GAME PORT B TIMER X

- 78

- GD3

- I/O

- 23

- GAME PORT B TIMER Y

- 79

- GD4

- I

- 24

- GAME PORT A BUTTON A

- 80

- GD5

- I

- 25

- GAME PORT A BUTTON B

- 81

- GD6

- I

- 26

- GAME PORT B BUTTON A

- 82

- GD7

- I

- 27

- GAME PORT B BUTTON B

- 83

- -SEL17X

- I/O

- 89

- IDE CD-ROM I/O PORT 170-177

- 84

- -SEL376

- I/O

- 90

- IDE CD-ROM I/O PORT 376

- 85

- -CDDENL

- O

- 88

- IDE CD-ROM LOW BYTE DATA

- ENABLE

- 86

- -CDDENH

### O   87 IDE CD-ROM HIGH BYTE DATA

### ENABLE

EXTERNAL GAME PORT, CD-ROM, AND

| ANALOG GROUND EXPANSION(CONT.) FIGURE 3.1 ALS100Plus+ PIN DIAGRAM SA7 SA6 SA5 SA4 SA3 SA2 SA1 SA0 -SEL376 GND -SEL17X -CDDENL -CDDENH SA8 -XRST -DACK0 DRQ0 IRQ11 XCLK IRQ10 81 98 99 97 96 95 94 93 92 91 90 89 88 87 86 85 84 83 82 ALS100Plus+ 100 SA9 1 80 IRQ5 SA10 2 79 IRQ7 SA11 3 78 DRQ1 SA12 4 77 -DACK1 SA13 5 76 GND SA14 6 75 XTALO SA15 7 74 XTALI SAEN 8 73 VDD SD0 9 72 MIDIOUT SD1 10 71 MIDIIN SD2 11 70 DRQ3 SD3 12 69 -DACK3 VDD 13 68 -SIOR SD4 14 67 -SIOW SD5 15 66 IRQ2/9 SD6 16 65 GIO2 SD7 17 64 GIO1 GND 18 63 GIO3 RESET 19 62 IRQ3 GD0 20 61 IRQ4 GD1 21 60 Item Pin Name Type Pin No. Description 87 -XRST O 86 CD-ROM OR FM RESET 88 | -FMCS I/O 58 EXTERNAL FM SYNTHESIZER CHIP SELECT 89 GIO3/ -BIOR I/O 63 GENERAL PURPOSE I/O SIGNAL 3 SYSTEM I/O READ STROBE 90 GIO1 I/O 64 |  | GENERAL PURPOSE I/O SIGNAL 1 91 GIO2 I/O 65 GENERAL PURPOSE I/O SIGNAL 2 POWER Item | Pin Name |  | Type Pin No. Description 92 VDD P 13 DIGITAL VCC, 5 OR 3.3 VOLT 93 VDD P 73 DIGITAL VCC, 5 OR 3.3 VOLT 94 AVDD1 P 28 ANALOG VCC, 5 OR 3.3 VOLT 95 AVDD2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100 P AGND P 38 43 ANALOG VCC, 5 OR 3.3 VOLT 96 GND P 18 DIGITAL GROUND 97 GND P 91 DIGITAL GROUND 98 GND P 76 DIGITAL GROUND 99 AGND P 54 ANALOG GROUND |  |  |  |  |  |  |  |  |

COMIRQ

GD2	    22									 5

| 9 -COMCS GD3 23 58 -FMCS GD4 24 57 SCLK GD5 25 56 SLRCK GD6 26 55 SDATA GD7 27 54 AGND AVDD1 28 53 MONO-I 32 31 31 32 31 31 31 34 35 36 37 38 39 40 41 42 43 44 45 46 47 48 49 50 33 32 31 ADL-I 29 52 ADR-I ADL-O 30 51 ADR-O CDR MIC DAL-O DAL-I CDL LINE-L ALEFT DAL-I DAL-I MUSIC-L SYNDACL-F AGND VREF2 MONO-O VREF-1 AVDD2 SYNDACR-F ARIGHT LINE-R DAR-I DAR-O MUSIC-R 4.0 Electrical Specification Analog Characteristics Assumptions: Ambient temperature（TA）--------------------------------------25℃ Voltage, Analog（VA）-------------------------------------------- +5.0 Voltage, Digital（VD）---------------------------------------------+5.0 Voltage, Filtered Digital（VFD）-------------------------------- +5.0 Input levels: Logical 0 --------------------------------------------------------- |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

----------0 V

Logical 1 --

| -----------------------------------------------------------------VD Input signal -----------------------------------------------1kHz sine wave Sample frequency（FS1）--------------------------------------- 48kHz Sample frequency（FS2）--------------------------------------- 8kHz Measurement bandwidth（16-bit linear coding）20Hz to 20kHz Parameter Min. Type Max. Units Analog Input Characteristics-Minimum Gain Setting(0 dB);unless otherwise specified ADC Resolution - 16 - Bits Instantaneous Dynamic Range（IDR） - 80 - dB Total Harm |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

onic Distortion（TH

| D） - 0.04 | - ％ Inter-Channel Isolation - 70 - dB Left/Right Channel Separation - 76 - dB Inter-Channel Gain Mismatch Line Inputs Mic Inputs - |  | - 0.5 0.5 dB dB | Programmable Input Gain Span 12.0 12.0 - dB Gain Step Size（32 levels） 1.3 1.5 1.7 dB ADC Offset Error - 10 100 |  | LSB Full Scale Input Voltage (MGE=1)Mic Inputs Line, CD-Audio, Music Inputs 0.26 2.6 0.28 2.8 0.31 Vpp Vpp (Table continued form previous page.) Gain Drift - 100 - ppm/℃ Input Resistance 47 - - kΩ Input Capacitance - |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - 15 pF Analog output Characteristics-Minimum attenuation(1 |  |  |  |  |  |  |  |  |

dB);unless otherwise specifie

| d DAC Resolution 16 - - Bits | Total Dynamic Range(TDR) All Outputs - 95 - dB |  | Instantaneous Dynamic Range(IDR) All Outputs - 80 - dB Total Harmonic Distortion(THD) - 0.04 - ％ Inter-Channel Isolation Line Out - 70 - dB Inter-Channel Gain Mismatch Line Out | - |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - 0.2 dB Voltage Reference Output-VREF - 2.00 - V Voltage Reference Output Current-VREF - 500 - | μA DAC Programmable Attenuation Span 46.5 46.5 - dB DAC Attenuation Step Size 0dB to -81dB -82.5dB to -94.5dB 1.3 1.0 1.5 1.5 1.7 2.0 dB dB DAC Offset Voltage - 10 50 mV Full Scale Output Voltage OLB=0 OLB=1 OUT, MOUT 1.8 2.6 2.0 2.9 2.25 3.2 Vpp |  |  |  |  |  |  |  |

Vpp

Gain Drift

-

100

-

ppm/℃

| Deviation From Linear Phase - - 10 degrees Mute Attenuation(0 dB) - 80 - dB Total Out-of-Band Energy 0.6xFS1 to 100kHz - | - -45 dB Audible Out-of-Band Energy 0.6xFS2 to 22kHz - -70 dB Signal To Noise Ratio 76 - - dB Output Power Rating-Main Out(4Ωspeakers) - - 2 |  | W/ |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

chnl

Power Supply

Power Supply Current         Digit

| al , Operating Analog, Operating Total Digital, Power Off Analog, Power Off - - - - - 61 41 102 100 100 110 60 170 300 300 mA mA mA μA μA Power Supply Rejection 1kHz 40 dB Note: These values are based on characterizations, not on production testing. Mixers Assumptions: Ambient temperature（TA）------------------- |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

------------

| ------- 25℃ Voltage, Analog（VA）-------------------------------------------- +5.0 Voltage, Digital（VD）---------------------------------------------+5.0 Voltage, Filtered Digital（VFD）-------------------------------- +5.0 Input levels: Logical 0 ---------------------------------------------------------------- 0 V Logical 1 ---------------------------------------------------------------- VD Input signal ---------------------------------------------1kHz sine wave Parameter Min. Type Max. Units Mixer Gain Range Span Line, CD-ROM, Music Monaural Master 45 42 42 46.5 45 45 - - - dB dB dB Step Size Line, CD-ROM, Music Monaural Master 1.3 1.3 1.3 1.5 1.5 1.5 1.7 1.7 1.7 dB dB dB Note: These values |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

are based on characterizatio

| ns, not on production testing. Maximum Ratings Assumptions: Analog Ground（AGND）-----------------------------------------------0 V Digital Ground（DGND）------------------------------------------------0 V Signal Ground（SGND）------------------------------------------------0 V All voltages measured with respect to:----------------------------------0 V Parameter Min. Max. Units Power supplies Digital(VD) Filtered Digital(VFD) Analog(VA) -0.3 -0.3 -0.3 6.0 6.0 6.0 V V V Total Power Dissipation (Supplies, Inputs, Outputs) - 1.0 W | Input Current per Pin (Except Supply Pins) -10.0 +10.0 mA Output Current per Pin (Except Supply Pins) |  | -50 +50 mA Analog Input Voltage -0.3 VA+0.3 V Digital Input Voltage -0.3 VD+0.3 V Operating Temperature Range 0 +70 ℃ Storage Temperature -50 +125 ℃ Electrostatic Protection (ESD)C=100pf, R=1.5KΩ 2000 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - V Warning: Operation beyond these limits may result in permanent damage to the ASIC normal operation is not guaranteed at these extremes. Timing Parameters Assumptions: Ambient temperature（TA）------------------------------------------- 25℃ Voltage, Analog（VA）------------------------------------------------- +5.0 - Voltage, Digital（VD）-------------------------------------------------- +5.0 Input levels: - Logical 0 --------------------------------------------------------------------- 0 V Logical 1 --------------------------------------------------------------------- VD Outputs loaded with: ------------------------------------------------------ 30pF Parameter Symbol Min. Max. Units OSC(14.31 MHZ) Frequency tOSCP 14.000 14.500 MHZ OSC High Width tOSCH 32 40 ns OSC Low Width tOSCL 32 40 ns SYSCLK Frequency - 8 9 MHz SYSCLK High Width - 50 70 ns SYSCLK Low Width - 55 70 ns -IOR/-IOW Command Width tCMD 120 - ns Write Data Setup To -IOW Rising tWDS 30 - ns Write Data Hold From -IOW Rising tWDH 15 - | ns Read Access Time tRAC 20 50 ns Address Setup To -IOR/-IOW Falling tAS 50 - ns Address Hold From -IOR/ |  |  |  |  |  |  |  |

-IOW Rising

tAH

30

-

ns

-DACK Setup To

| -IOR/-IOW Falling tDKS 40 - ns -DACK Hold From -IOR/-IOW Rising tDKH 160 - ns SD Hold Form -IOR Rising tDH 0 20 ns XTAL Frequency - - 14.318 MHz XTAL High Time - 18 - ns XTAL Low Time - 18 - ns Sample Frequency - 3.918 50 kHz DRQ Hold Form -IOR/ -IOW Falling tDRH 0 25 ns Note: These values are based on characterizations, not on production testing. Operating Environment Assumptions: Analog Ground（AGND）---------------------------------------- 0 V Digital Ground（DGND）----------------------------------------- 0 V Signal Ground（SGND）----------------------------------------- 0 V All voltages measured with respect to: -------------------------- 0 V Parameter Min. Type Max. Units Power supplies Digital(VD) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Filtered Digital(VFD)

Analog(VA

| ) 4.75 4.75 4.75 5.0 5.0 5.0 5.25 5.25 5.25 V V V Operating Ambient Temperature(TA) 0 25 70 ℃ | 5.0 ALS100Plus+ Physical Description 100 81 1 80 Pd L2 L1 Pw 51 30 31 50 H1 W1 W2 q Hp H2 L3 Unit: mm Symbol Min. Type Max. L1 |  | 24.45 24.80 25.12 L2 19.75 20.00 | 20.25 L3 1.00 1.20 1.40 W1 18.45 18.80 19.15 W2 13.75 14.00 14.25 H1 2.70 3.00 3.30 H2 0.11 0.51 |  | 0.91 Hp 0.04 0.15 0.26 Pw 0.50 0.65 0.80 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pd 0.18 0.30 0.42 q 0° 5° 10° µ §µ § 6.0 Design Example ALS200/100+ ISA SOUND w/ IDE CD INTERFACE Revised: January 20, 1997 AS9705 (1623B) Revision: A Bill Of Materials January 20, 1997 8:32:12 Item Quantity Reference Part 1 1 CN1 DB15 D-SUB 15PIN FEMALE 2 6 C1 |  |  |  |  |  |  |  |  |

10u  	10UF 16V TANT CAP

C18                    	10u      	10

| UF 16V TANT CAP C24 10u 10UF 16V TANT CAP C36 10u 10UF 16V TANT CAP C44 10u 10UF 16V TANT CAP C52 10u 10UF 16V TANT CAP 3 1 C32 100P 100PF 0805 SMD CAP 4 19 C2 .1U .1UF 0805 SMD CAP C3 .1U .1UF 0805 SMD CAP C5 .1U .1UF 0805 SMD CAP C6 .1U .1UF 0805 SMD CAP C7 .1U .1UF 0805 SMD CAP |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

C8

| .1U .1UF 0805 SMD CAP C11 .1U .1UF 0805 SMD CAP C15 .1U .1UF 0805 SMD CAP C20 .1U .1UF 0805 SMD CAP C28 .1U .1UF 0805 SMD CAP C29 .1U .1UF 0805 SMD CAP C30 .1U .1UF 0805 SMD CAP C31 .1U .1UF 0805 SMD CAP C33 .1U .1UF 0805 SMD CAP C34 .1U .1UF 0805 SMD CAP C35 .1U .1UF 0805 SMD CAP C40 .1U .1UF 0805 SMD CAP C47 .1U .1UF 0805 SMD CAP |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

C54                    	.1U      	.1UF 0805 SMD CAP

5      10    	C4                     	100U 	100UF 16V TANT CAP

C17                    	100U     100UF 16V TANT CAP

C37                    	100U     100UF 16V TANT CAP

C38                    	100U     100UF 16V TANT CAP

C39                    	100U     100UF 16V TANT CAP

C41                    	100U     100UF 16V TANT CAP

C43                    	100U     100UF 16V TANT CAP

C45                    	100U     100UF 16V TANT CAP

C46                    	100U     100UF 16V TANT CAP

C55                    	100U     100UF 16V TANT CAP

6       3    	C12                    .22U   	.22UF 0805 SMD CAP

C48                    	.22U     .22UF 0805 SMD CAP

C56                    	.22U     .22UF 0805 SMD CAP

7       2    	C9                  	4.7P      4.7PF 0805 SMD CAP

C10                    	4.7P      4.7PF 0805 SMD CAP

8      10    	C13                    	1000P    1000PF 0805 SMD CAP

C14                    	1000P    1000PF 0805 SMD CAP

C16                    	1000P    1000PF 0805 SMD CAP

C19                    	1000P    1000PF 0805 SMD CAP

C21                    	1000P    1000PF 0805 SMD CAP

C22                    	1000P    1000PF 0805 SMD CAP

C23                    	1000P    1000PF 0805 SMD CAP

C25                    	1000P    1000PF 0805 SMD CAP

C26                    	1000P    1000PF 0805 SMD CAP

C27                    	1000P    1000PF 0805 SMD CAP

9    	1    	C51                   	220U     220UF 16V RAD ELECT CAP

10     3    	C42                   	470U     470UF 16V RAD ELECT CAP

C49                   	470U     470UF 16V RAD ELECT CAP

C50                   	470U     470UF 16V RAD ELECT CAP

11		5	    	C53                   	.01U		.01UF 0805 SMD CAP

C57                   	.01U     .01UF 0805 SMD CAP

C58                    	.01U     .01UF 0805 SMD CAP

C59                   	.01U     .01UF 0805 SMD CAP

C60                   	.01U     .01UF 0805 SMD CAP

12     1    	JP1                   	MITSUMI  4PIN 2MM AUDIO HEADER

13     1    	JP2                    	IDE     	 4PIN 0.1” AUDIO HEADER

14     1    	JP3                    	CREATIVE 4PIN 0.1” AUDIO HEADER

15     1    	JP4                   	2X1       2X1 HEADER (PC SPEAKER)

#### 16     1    	JP5A                   3X2       3X2 HEADER

17     1    	JP5B                   2X2       2X2 HEADER

- 18     1    	J1                    	20X2      20X2 DUAL ROW HEADER

- 19     1    	J2                     	13X2      13X2 DUAL ROW HEADER

- 20     2    	**L1, L2                          	 **SHORT

- 21     3    	PH1                   	PHONEJACK	SJ-100 5PIN PHONEJACK

- PH2                   	PHONEJACK 	SJ-100 5PIN PHONEJACK

- PH4                   	PHONEJACK 	SJ-100 5PIN PHONEJACK

- 22     6    	R8                    	4.7K   		4.7K OHM 0805 SMD RES

- R9                    	4.7K        	4.7K OHM 0805 SMD RES

- R11                    	4.7K        	4.7K OHM 0805 SMD RES

- R12                    	4.7K        	4.7K OHM 0805 SMD RES

- R13                    4.7K        	4.7K OHM 0805 SMD RES

- R14                    	4.7K        	4.7K OHM 0805 SMD RES

- 23     2    	**R17                   10K         	10K OHM 0805 SMD RES

- **UNMOUT IF USING 6502

- EMULATION

- **R2                    10K         	10K OHM 0805 SMD RES

- **UNMOUNT IF

- USING EXTERNAL OPL3

- 24     7    	**R3, R6                33           	33 OHM 0805 SMD RES

- **FOR ALS200 ONLY

- **R4                   	33          	33 OHM 0805 SMD RES

- **DO NOT MOUNT IF

- USING CRYSTAL

- **R5                    33          	33 OHM 0805 SMD RES

- **FOR ALS100+ ONLY

| R10 33 33 OHM 0805 SMD RES R15 33 33 OHM 0805 SMD RES R16 33 33 OHM 0805 SMD RES 25 4 R7 47 47 OHM 0805 SMD RES R18 47 47 OHM 0805 SMD RES R19 47 47 OHM 0805 SMD RES R31 47 47 OHM 0805 SMD RES 26 3 R21 100 100 OHM 0805 SMD RES R28 100 100 OHM 0805 SMD RES R23 100 100 OHM 0805 SMD RES 27 1 R20 1K 1K OHM 0805 SMD RES 28 1 R1 1.8M 1.8M OHM 0805 SMD RES 29 9 R37 1.2K 1.2K OHM 0805 SMD RES R39 1.2K 1.2K OHM 0805 SMD RES R42 1.2K 1.2K OHM 0805 SMD RES R44 1.2K 1.2K OHM 0805 SMD RES R46 1.2K 1.2K OHM 0805 SMD RES R47 1.2K 1.2K OHM 0805 SMD RES R49 1.2K 1.2K OHM 0805 SMD RES R50 1.2K 1.2K OHM 0805 SMD RES R52 1.2K 1.2K OHM 0805 SMD RES 30 11 R22 100K 100K OHM 0805 SMD RES R23 100K 100K OHM 0805 SMD RES R24 100K 100K OHM 0805 SMD RES R25 100K 100K OHM 0805 SMD RES R26 100K 100K OHM 0805 SMD RES R27 100K 100K OHM 0805 SMD RES R32 100K 100K OHM 0805 SMD RES R40 100K 100K OHM 0805 SMD RES R41 100K 100K OHM 0805 SMD RES R48 100K 100K OHM 0805 SMD RES R51 100K 100K OHM 0805 SMD RES 31 2 **R25, R30 4.7M **RESERVED 32 2 R35 68K 68K OHM 0805 SMD RES R38 68K 68K OHM 0805 SMD RES 33 2 R43 2K 2K OHM 0805 SMD RES R45 2K 2K OHM 0805 SMD RES 34 2 R34 75K 75K OHM 0805 SMD RES R36 75K 75K OHM 0805 SMD RES 35 4 **R53,R54,R55,R56 **RESERVED 36 2 D1 1N4148 1N4148 THRU-HOLE D2 1N4148 1N4148 THRU-HOLE 37 1 **U4 YMF262 OPL 3 ** OPTIONAL OPL3 38 3 **U1 74LS245 20PIN SOIC **FOR OPTIONAL OPL3 U2 74LS245 20PIN SOIC U3 74LS245 20PIN SOIC 39 1 U5 ALS200/ ALS200/100+ SOUND CHIP ALS100+ 40 1 U7 LM7805CT TO-220 5V VOLTAGE REGULATOR 41 1 U6 TEA2025B 14PIN DIP 42 1 **Y1 14.318MHz 14.31818MHZ SERIES CRYSTAL |  |  |  |  |
| --- | --- | --- | --- | --- |

**DO NOT MOUNT IF

USING

MOTHERBOARD OSC