Conducted Common Mode Disturbance Noise Test at Telecommunication ports in CISPR 22 – About the Realtek’s Ethernet and Fast Ethernet Product

After Aug. 1st, 2001, all telecommunication devices sell to the Europe need to

pass the conducted disturbance regulation as stated on the CISPR 22-1998.

Limits for conducted disturbance at mains terminals and telecommunication ports:

The equipment under test (EUT) shall meet the limits in tables 1 and 3 or 2 and 4, as applicable, including the average limit and the quasi-peak limit when using, respectively, an average detector receiver and quasi-peak detector receiver and measured in accordance with the methods described in clause 9 of CISPR 22. Either the voltage limits or the current limits in table 3 or 4, as applicable, shall be met except for the measurement method of C.1.3 of CISPR 22 where both limits shall be met. If the average limit is met when using a quasi-peak detector receiver, the EUT shall be deemed to meet both limits and measurement with the average detector receiver is unnecessary.

If the reading of the measuring receiver shows fluctuations close to the limit, the reading shall be observed for at least 15 s at each measurement frequency; the higher reading shall be recorded with the exception of any brief isolated high reading which shall be ignored.

For the case of Ethernet, the transmit waveform is a Manchester encoded differential pulse passing through a 3-pole low-pass Butterworth filter. Since the transmit clock is 10Mhz, if transmitting continues 1s or 0s in the data field, thus will cause 10Mhz radiation/conduction noise increase. As we saw on the fig2 and fig3 shows the cases we stated above.

As for Fast Ethernet, the basic transmit clock is 125Mhz. But for the process of transferring from NRZ to NRZI and passing to MLT3 driver, continues 1s (such as IDLE) will form a 62.5Mhz spectral fundamental. Then the scramble process will scramble the data so there won’t be many continues 1s on the transmit symbols and no peak spectral on 62.5Mhz and the energy will be re-distributed to the lower frequency band. As shown on fig4, we may see that most of the power spectral lay on the 6~30Mhz band.

The Ethernet (10M) and Fast Ethernet (100M) signals both have significant spectral density in the frequency range of 6 MHz to 30 MHz. According to CISPR 22, Table 4, Note 3, a relaxation of 10 dB over the frequency range of 6 MHz to 30 MHz is allowed for high speed services having significant spectral density in this band. Thus we may apply this to our Ethernet/Fast Ethernet products.

8139C

fig1:10M waveform,Data pattern: Random		fig2:10M waveform, Data pattern: All ones

fig3: 10M waveform, Data pattern : All zero		fig4: 100M waveform, Data pattern : Random

8019

fig5: 10M waveform, Data pattern : Random		fig6: 10M waveform, Data pattern : All ones

fig7: 10M waveform, Data pattern : All zero