Before you upgrade, there are something you should know.

- Write down all the current settings as you have to enter them manually after upgrade.

- Choose the firmware which has the same hardware version as yours.

- DO NOT upgrade firmware from a computer with a wireless connection. Use a computer that you can connect to the router with an Ethernet cable. You may damage the router due to the increased packet loss with a wireless connection.

- DO NOT turn off the power during upgrading.

Step 1 Log into the ADSL Router by typing http://192.168.1.1 in the address field of the browser (such as Internet Explorer, Netscape). Both the User Name and Password are admin unless you have changed it.

Step 2 Click Management -> Update Software on the left, click the Browse button to choose the firmware you downloaded before.

Step 3 Click the Update Software button, Wait for about 5 minutes, and your DSL Router will reboot.

Step 4 Verify that the router’s firmware is upgraded.

Step 5 Reenter the settings you have noted down before upgrading.